SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 1- CONSTRAINT-BASED PROBLEM SOLVING

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO5 | Assessment: | ASSESSMENT TOOL1- Constraint-Based Problem Solving |
| Weightage: | 40% | Total Marks: | 25 |
| CO5 Statement: | Construct I/O communication mechanisms by comparing memory-mapped and I/O-mapped techniques, interrupt handling (vectored, hardware, software), DMA controllers, and advanced bus interfaces including PCIe, USB, NVMe, and Thunderbolt. |  |  |
| Student Name: | ARUN PANDIAN K | Reg.No.: | 192511086 |
| Department: | CSE | Date: | 27/8/26 |

Code of Conduct:

I ARUN PANDIAN K (192511086) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student: ARUN PANDIAN K

ANNEXURE A

Constraint-Based Problem Solving

1.Construct an I/O communication mechanism for a real-time embedded system with the following constraints:

- CPU utilization must be below 15%

- Multiple sensors generate interrupts every 1 ms

- Use vectored interrupt handling

- Select between memory-mapped and I/O-mapped I/O for optimal latency

Answer:

Problem Understanding: The task is to design an I/O communication mechanism for a real-time embedded system that keeps CPU utilization below 15% while multiple sensors generate interrupts every 1ms, using vectored interrupt handling, and choosing between memory-mapped and I/O-mapped addressing for optimal latency.

Constraint Analysis: At a 1ms inter-interrupt period, sensors collectively generate on the order of 1000 interrupts/second; with a 15% CPU budget, that allows roughly 150 microseconds of CPU time per second-worth of interrupts to be spent per interrupt on average – a very tight per-event budget that rules out any dispatch mechanism with significant per-event overhead. I/O-mapped I/O (separate IN/OUT-style instructions and address space, as in x86) adds instruction-decode and address-translation overhead on every register access, while memory-mapped I/O lets sensor registers be accessed with ordinary load/store instructions at normal memory speed – directly reducing per-interrupt CPU cost. A non-vectored (polled/daisy-chain) interrupt scheme would also require the CPU to query each sensor in turn to find the interrupting device, adding O(n) overhead as sensor count grows, which the vectored-interrupt constraint is specifically meant to avoid.

Solution Development: Use memory-mapped I/O for all sensor control/status/data registers, since most modern embedded cores (e.g., ARM Cortex-M) do not even provide a separate I/O address space, making memory-mapped I/O both the lower-latency and the more portable choice. Use a vectored interrupt controller (e.g., an NVIC-style controller) where each sensor's interrupt line maps directly to a dedicated entry in the interrupt vector table, giving hardware-resolved, near-constant-time dispatch to the correct ISR rather than software polling. Keep each ISR minimal – read the sensor's data register into a buffer and set a flag or trigger a lightweight follow-up – deferring any heavier processing to a lower-priority background task, so that the CPU time actually spent inside interrupt context per event stays well under the ~150μs/event ceiling implied by the 15% budget.

Application of Concepts: This applies memory-mapped I/O addressing, vectored/NVIC-style interrupt controllers, real-time interrupt-latency budgeting, and the ISR/deferred-task (top-half/bottom-half) processing pattern.

Justification: Memory-mapped I/O is justified over I/O-mapped I/O because it removes the extra instruction-set overhead of separate I/O instructions, directly lowering the per-interrupt CPU cost that the tight 15% budget demands; vectored interrupt handling is justified because it gives deterministic, hardware-resolved dispatch instead of linear polling, which is essential once multiple sensors are interrupting at a combined ~1kHz rate, since polling overhead would grow with sensor count and threaten both the latency and CPU-utilization constraints simultaneously.

2.Design an I/O subsystem under these constraints:

- High-speed storage requires throughput > 3 GB/s

- Must use NVMe over PCIe

- CPU should not handle bulk data transfer

- Integrate DMA controller with interrupt notification

Answer:

Problem Understanding: The task is to design a high-speed storage I/O subsystem delivering throughput above 3GB/s, mandated to use NVMe over PCIe, with the constraint that the CPU must not handle bulk data transfer directly, and that a DMA controller must be integrated with interrupt-based completion notification.

Constraint Analysis: A throughput requirement above 3GB/s immediately rules out older interfaces such as SATA (capped around 600MB/s); a PCIe Gen3 x4 link provides roughly 3.94GB/s of raw bandwidth (about 985MB/s per lane), comfortably clearing the requirement, with PCIe Gen4 x4 providing roughly double that headroom if needed. The constraint that the CPU must not handle bulk transfer rules out programmed I/O (where the CPU copies each word between device and memory) and mandates DMA; NVMe as a protocol is built around exactly this model, using submission/completion queue pairs and PRP/SGL (scatter-gather) descriptors so the storage controller's DMA engine moves data directly to/from system RAM without per-word CPU involvement.

Solution Development: Connect an NVMe SSD via a PCIe Gen3 x4 (or Gen4 x4 for additional headroom) link. The CPU's only involvement is to set up an I/O request by writing a command (including PRP/SGL descriptors describing the memory buffer) into an NVMe submission queue; the SSD's DMA engine then transfers the actual data directly to/from system RAM without further CPU copying. On completion, the controller writes a completion queue entry and raises an MSI-X (Message Signaled Interrupt) to notify the CPU, so the CPU is free to perform other work throughout the transfer and only reacts once at completion.

Application of Concepts: This applies PCIe lane bandwidth calculation, the NVMe submission/completion queue and PRP/SGL architecture, DMA-based bulk transfer, and MSI-X interrupt-driven completion notification.

Justification: PCIe Gen3/4 x4 lanes are justified by the direct bandwidth-versus-requirement comparison (3.94GB/s+ available against a 3GB/s requirement); NVMe-over-PCIe with DMA is justified because it is specifically designed to eliminate CPU involvement in bulk data movement, unlike older AHCI/SATA command models; and MSI-X completion interrupts are justified over status-register polling because polling would itself consume CPU cycles throughout the transfer, contradicting the 'CPU should not handle bulk data transfer' constraint even indirectly.

3.Construct a multi-device communication architecture with constraints:

- 6 peripherals using USB, 2 high-speed devices using Thunderbolt

- Avoid interrupt starvation

- Use hardware and software interrupts

- Ensure fair bus arbitration

Answer:

Problem Understanding: The task is to construct a multi-device communication architecture serving 6 USB peripherals and 2 Thunderbolt high-speed devices, while avoiding interrupt starvation, using both hardware and software interrupts, and ensuring fair bus arbitration.

Constraint Analysis: USB is a host-scheduled bus where the host controller allocates bus time across frames/microframes; with 6 peripherals sharing a controller, a bandwidth-heavy device could dominate the schedule and starve others if scheduling is not fair. Thunderbolt tunnels PCIe (and DisplayPort) traffic over a high-speed serial link and supports daisy-chaining with its own bandwidth arbitration; keeping the 2 high-speed devices on Thunderbolt, separate from the 6 USB peripherals, prevents the higher-bandwidth Thunderbolt traffic from competing directly with USB's frame-scheduled traffic for the same bus resource.

Solution Development: Assign the 6 USB peripherals to a shared xHCI host controller using its native round-robin/time-sliced frame scheduling, which guarantees each device a minimum share of frame time rather than allowing one device's bulk transfers to monopolize the bus. Use hardware interrupts for time-sensitive, asynchronous device events, dispatched through the system interrupt controller with a priority scheme that ages or rotates priorities (rather than pure fixed-priority) specifically so a continuously-active high-priority device cannot indefinitely starve lower-priority ones. Keep hardware ISRs minimal, deferring the bulk of per-event processing to software-interrupt-triggered background routines (bottom halves) so no single device's ISR can block servicing of another device's interrupt. On the Thunderbolt side, the 2 high-speed devices share the PCIe-tunneled link using standard PCIe weighted round-robin arbitration between their virtual channels, guaranteeing each a fair bandwidth allocation.

Application of Concepts: This applies USB xHCI frame-based scheduling, Thunderbolt/PCIe tunneling with virtual-channel arbitration, interrupt-priority aging schemes, and software interrupts for deferred (bottom-half) processing.

Justification: Separating the 6 USB peripherals (frame-scheduled, round-robin) from the 2 Thunderbolt devices (independently arbitrated over their own tunneled link) is justified because it prevents the two very different traffic classes – numerous lower-bandwidth USB devices versus few high-bandwidth Thunderbolt devices – from contending for the same scheduling resource; combining short hardware ISRs with software-interrupt-triggered deferred processing is the standard, justified technique for preventing interrupt starvation, since it ensures no single device's handler can block the timely servicing of another device's pending interrupt.

4.Design a data acquisition system with constraints:

- Continuous data stream at 500 MB/s

- Minimal CPU intervention required

- Must compare DMA transfer vs interrupt-driven I/O

- Use memory-mapped I/O for device registers

Answer:

Problem Understanding: The task is to design a data acquisition system sustaining a continuous 500MB/s data stream with minimal CPU intervention, requiring an explicit comparison between DMA transfer and interrupt-driven I/O, and using memory-mapped I/O for the device's control/status registers.

Constraint Analysis: At 500MB/s, even with 4-byte transfer units, the system would need roughly 125 million transfers per second; interrupt-driven (programmed) I/O incurs an interrupt entry/exit overhead typically in the range of hundreds of nanoseconds to a few microseconds per event, which caps achievable interrupt-driven throughput at perhaps 1 million interrupts/second even under optimistic assumptions – roughly two orders of magnitude short of what 500MB/s of per-word interrupt-driven transfer would require. This makes pure interrupt-driven data movement infeasible for the bulk data path and mandates DMA.

Solution Development: Use a DMA controller configured for continuous scatter-gather / ring-buffer transfer, streaming acquisition data directly from the device into a circular buffer in system RAM without CPU involvement per sample. Employ double-buffering (ping-pong buffers) so the CPU is interrupted only once per buffer-half or buffer-full event rather than once per sample – collapsing millions of potential per-sample interrupts down to a small, manageable interrupt rate while still sustaining 500MB/s of throughput. Device control and status registers (start/stop, buffer base/length, error flags) are accessed via memory-mapped I/O, letting the CPU configure and monitor the acquisition hardware with ordinary load/store instructions, separate from the DMA-driven bulk data path.

Application of Concepts: This applies DMA scatter-gather/ring-buffer streaming, double-buffered interrupt coalescing, and memory-mapped I/O for the low-frequency control-plane path as distinct from the high-throughput data-plane path.

Justification: Comparing the two approaches directly justifies the choice: interrupt-driven I/O requires CPU involvement on every transfer unit, making it fundamentally unable to sustain 500MB/s without saturating the CPU purely on interrupt-handling overhead, whereas DMA with double-buffering moves the bulk of the data with only periodic, buffer-level interrupts – the only approach that can simultaneously satisfy the throughput and minimal-CPU-intervention constraints; memory-mapped I/O is justified for the control/status path since it is simple and fast for the comparatively rare configuration and status accesses, which do not need DMA's bulk-transfer optimizations.

5.Construct an interrupt handling mechanism with constraints:

- Support nested interrupts

- Maximum interrupt latency < 10 µs

- Use vectored interrupts for high-priority devices

- Use software interrupts for background tasks

Answer:

Problem Understanding: The task is to construct an interrupt handling mechanism supporting nested interrupts, bounding maximum interrupt latency below 10 microseconds, using vectored interrupts for high-priority devices, and software interrupts for background tasks.

Constraint Analysis: A 10μs latency ceiling must cover interrupt controller arbitration, context save, vector lookup, and the jump to the ISR; any scheme requiring the CPU to poll or daisy-chain through devices to identify the interrupt source introduces variable, potentially unbounded latency that could violate this budget, whereas a hardware vectored controller resolves the correct ISR address in a small, fixed number of cycles, keeping dispatch latency both low and predictable. Supporting nested interrupts additionally requires the controller to support multiple priority levels and allow a higher-priority interrupt to preempt an in-progress lower-priority ISR, rather than forcing all interrupts to wait for the current handler to finish.

Solution Development: Use a priority-based vectored interrupt controller (e.g., an NVIC-style design) configured with multiple priority levels, assigning high-priority devices to the highest levels so they receive minimal dispatch overhead and can preempt any lower-priority ISR currently executing – satisfying both the nesting requirement and the <10μs latency budget. Keep every ISR extremely short (minimal register save/restore, no blocking operations) so that no single handler can run long enough to delay a pending higher-priority interrupt beyond the 10μs ceiling, since a nested-capable controller only bounds latency well if ISR execution times themselves stay bounded. Route non-time-critical background work through software interrupts (e.g., an SVC/trap instruction or deferred-procedure-call mechanism) triggered at the tail of a hardware ISR, running at a priority strictly lower than any hardware interrupt so background processing never delays real-time device servicing.

Application of Concepts: This applies vectored, priority-based interrupt controllers with preemption, interrupt nesting design, worst-case ISR latency budgeting, and software interrupts for deferred/background task scheduling.

Justification: Vectored interrupts are justified as the only way to reliably meet the <10μs constraint, since hardware-resolved dispatch is both fast and deterministic compared to any polling-based alternative whose worst-case latency grows with device count; multiple priority levels with preemption are justified because they are what actually enables interrupt nesting, letting an urgent event interrupt a lower-priority handler already running rather than queueing behind it; and offloading background work to low-priority software interrupts is justified because it keeps non-urgent processing from ever competing with the hard real-time latency budget reserved for high-priority hardware events.

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 25% | Clearly defines the problem and identifies all relevant constraints accurately | Identifies most constraints with minor gaps | Partial understanding; misses key constraints | Fails to identify constraints correctly |
| Constraint Analysis | 25% | Analyzes constraints effectively and prioritizes them logically | Provides reasonable analysis with some prioritization | Limited analysis; weak prioritization | No meaningful analysis or prioritization |
| Solution Development | 25% | Develops optimal and feasible solutions satisfying all constraints | Develops workable solutions with minor limitations | Solutions partially satisfy constraints | Solutions do not meet constraints |
| Application of Concepts | 15% | Effectively applies appropriate concepts, methods, or tools | Applies concepts with minor errors | Limited application; requires guidance | Unable to apply relevant concepts |
| Justification | 10% | Provides strong justification for decisions with logical reasoning | Provides justification with some clarity | Weak or unclear justification | No justification provided |